|  | Eastern Mediterranean University Department of Mechanical Engineering Laboratory Handout |
| --- | --- |

COURSE:    Thermal Power Engines (MENG446) –Group1

Semester: Spring (2017-2018)

Name of Experiment: The Electric Steam Generator- Take home experiment

Submission Due : 14th May Monday 2018

Lab Assistant: Gorkem Ozankaya (e-mail: gorkem.ozankaya@cc.emu.edu.tr)

Instructor: Asst. Prof. Dr. Devrim Aydin (e-mail: devrim.aydin@emu.edu.tr)

Submitted by:

Student No:

Group No:

Date of experiment:

Date of submission:

------------------------------------------------------------------------------------------------------------

EVALUATION

| Data , Results & Graphs (50%) |  |
| --- | --- |
| Discussion, Conclusion & Answer to Questions (30%) |  |
| Neat and tidy report writing (20%) |  |
| Overall Mark |  |

Honor Pledge:

By submitting this report I pledge that I have neither given nor received unauthorized assistance on this assignment.

__________                             ______________

Date                                         Signature

- Objective and Introduction

The objective of the steam generator experiment is to practice thermodynamics of evaporation of water and to analyze energy requirement for liquid-gas phase change of saturated steam.

- Experimental procedure

- Use an electric kettle for boiling the water.

- Fill the kettle up to a certain volume (between 1-2 liter)

- Turn on the electricity

- Measure and record the operational time of the kettle till boiling.

- Activities

  - Determination of the power rating of the kettle

- Please assume initial temperature (before boiling) T1=20 °C and final temperature (after boiling) T2= 100 °C.

- Measure the weight of the water before boiling process and neglect the weight loss of water during the boiling process.

- Assume P=constant at 1 atm during the boiling process.

- Assume that in the final state, water is in saturated mixture state and the quality X=10%.

- Calculate specific enthalpy of compressed liquid water for initial state (h1) by using equation h =  cp. T, and calculate the enthalpy of saturated mixture by reading hf and hfg values from table A4 for the given quality value.

- Measure the kettle operational time during the boiling process.

- Calculate the power rating of kettle by using the equation below;

Please show / describe all steps used for determination of initial and final enthalpies, measurement of time and calculation of water mass.

welectric=mwater×h2-h1∆t

- Show the process on a T-v diagram. Please insert the T, P and v  values in the diagram