EASTERN MEDITERRANEAN UNIVERSITY

(2022-23/1)

ARCH 342 - Working Drawing

Instructor:Prof. Dr. Halil Z. Alibaba

Course Asistant: Abubakar Dada Dauda

Checklist for Drawings

A-) Site Plan (1/100)

1- Top view of the Project

2- Axis

3- Roof Slopes(Represented with dotted lines) & Slope percentage indications (%2/%3)

4- Rainwater pipes

5- Roof insulation & Terrace covering material information

6- Wastewater System drawings (to Septic Tank/Drainage Well/Sanitation)

7- Levels

8- Dimensions

i- Length & Width of the Whole Building

ii- intersection of AXIS (Building) to Corner (Site)

iii- Perpendicular distance from façade to the site (from the façades that are parallel to site)

iV- Dimensions detailing the terraces

9- Car Park(s)

10- Pedestrian Access & Car Access Triangles

11- Section Lines

12- Site Codes

13- Section representations (Showing only the reinforced concrete) (1/100)

14- Garden Wall/Fence Sections (1/20)

15- North Sign

16- Legend

17- Neighbour plot names.

Note: Wet space tiling!

B-) Floor Plans (1/50)

1- Axis (having one vertical one horizontal passing each column)

2- Columns (hatching)

3- Walls

4- Windows & Doors (with window sills) with 1/50 drawing technique

5- Fixed Furniture wc, kitchen, wardrobes,

6- Garden elements (only at the ground floor)

7- Name, Area & Level of Spaces (all different heights require annotation with levels)

8- Waste water & Rainwater Pipes (shafts? external rainwater pipes)

9- Codes of the windows and doors

10- Codes of the stairs & steps

11- Interior and Exterior Dimensions

12- Section Lines

13- Table of spaces information (covering material of floor/walls/ceiling)

14- North Sign

15- Legend

16- Dash and Dot lines.

Note: Wet space tiling!

C-) Sections (1/50)

*one section will pass through the stairs

*the other section must have 90 degree alignment with the staircase section line (

*in other words if one section is vertical the other one has to be horizontal!

1- Axis

2- Reinforce Concrete (Structural Elements)

3- Walls that are cut by the section line

4- Walls that are not cut by the section line (Drawn with Elevation Front,Middle, Behind layers)

5- Windows (Glass Drawing will be included) & Doors

6- Fixed Furniture (drawn with furniture layer)

7- Foundation and earth drawing (concrete, foundation, blocks, compacted earth,etc...)

8- Floor covering drawing on top of concrete slab

9- Slab section information (floor material & ceiling material & roof material & foundation material)

10- Names of the spaces

11- Dimensions (showing lintel/window/beam height & floor to floor height)

12- Levels

13- Levels of doors and windows behind the section

14- Legend

Note: Wet space tiling!

D-) Elevations (1/50)

1- Side View of the project with extensions of mass and void of project

2- Height and radius of arcs, if any

3- Chimney, if any

4- Garden elements which are close to the building (names and levels)

5- Basement Floor shown with dash lines, if applicable

6- Axis

7- Dimension between the axis

8- Information about window and door opening directions (dash lines for regular, arrows for sliding, plus sign for fixed)

9- Rain water pipes & information

10- Slap and Concrete representations behind the façade (w/ dash lines)

11- Surface Covering Material Information

12- Information about railings, balustrades, shading elements and other fixed elements (height and material)

13- Levels

14- Legend

E-) Detail Drawings of Stairs (1/20)

1- Partial Floor Plans that show the stairs on each floor, including close surroundings. Additional to 1/50, these details will be added:

i- Axis with scaled numbers and letters (%40 of 1/50)

i- Screed (3cm) representations with screed layer

iii- Levels according to 1/20 scale (%40 of 1/50)

iv- Dimensions and names of spaces according to 1/20 scale (%40 of 1/50)

v- Stair number, dimensions and flight direction according to 1/20 scale (%40 of 1/50)

vi- Boundary of the plans with a thick dash line

vii- Section Line

2- Partial Section(s) that show the stairs from foundation to roof, including its close. Additional to 1/50, these details will be added:

i- Axis with scaled numbers and letters according to 1/20 scale (%40 of 1/50)

i- Screed (3cm) and floor cover representations with screed layer

iii- Levels according to 1/20 scale (%40 of 1/50)

iv- Stair numbers, Dimensions, and names of spaces according to 1/20 scale (%40 of 1/50)

v- Boundary of the section with a think dash line

vi- Detail representations of railings and balustrades including information about the height and material